Unit 9 -  Tangencies and Links

9.1. 	Tangencies

Tangency is the most common geometric way to smoothly connect lines and surfaces. It is used very often in many designs, from architectonic structures to decorative forms and ordinary objects.

Remember that two lines or two surfaces are tangent when they have a single point in common and they do not intersect.

There are different procedures to tangentially connect any two circumferences or a straight line and a circumference. All of them find the tangency points by using the following properties and geometrical considerations:

Properties

- “When a straight line is tangent to a circumference, the radius is perpendicular to the line at the tangency point”.

- “When two circumferences are tangent (exteriorly or interiorly), the contact point is on the straight line that connects both centres”.

Geometric considerations

- “When a circumference passes on two points A and B, its centre is placed on the perpendicular bisector of the segment AB”.

- “When a circumference is tangent to two straight lines, its centre is placed on the bisector of the angle defined by both lines”.

In the textbook (pages 126 and 127) you will find the different procedures to draw tangents to:

○   one circumference:

- at a given point

- parallel to a given direction

- from a point outside the circumference

○○  two circumferences:

- exterior tangents

- interior tangents

9.2. 	Links  in  Technical  Curves

A link is the connection between two or more lines to become only one by finding their tangency points.

Ovals and ovoids belong to the kind of links called closed, because they start and end at the same point, while many spirals are open links.

Ovals and ovoids are also called circular curves because they are made of tangent arcs of circumference.

- Oval

A flat, convex, closed curve shaped like a stretched circle. It is formed by four tangent arcs of circumference whose centres are on two perpendicular symmetry axes.

- Ovoid

A flat, convex, closed curve shaped like an egg. It is also formed by four tangent arcs of circumference, but it has only one axis of symmetry.

- Spiral

A flat, open curve generated by a point rotating around a fixed centre and gradually increasing the radius. The radial distance between two consecutive rotations is called the spiral’s step.

An important kind of spiral is formed by the volutes or polygonal core spirals. They are made of tangent circular arcs whose centres are the vertices of a central regular polygon.